2190151 Computer Programming Lab

Lab 5: Weather Station

Objectives:

1. Student be able to use simple sensor in their IoT project

2. Student be able to write graphical display of data on M5Stack

3. Student be able to create Hygrometer, Temp Clock

Background Theory

The DHT11Temperature & Humidity Sensor features a temperature& humidity sensor complex with a calibrated digital signal output. By using the exclusive digital-signal-acquisition technique and temperature & humidity sensing technology, it ensures high reliability and excellent long-term stability. This sensor includes a resistive-type humidity measurement component and an NTC temperature measurement component, and connects to a high performance 8-bit microcontroller, offering excellent quality, fast response, anti-interference ability and cost-effectiveness.

Task1: Basic weather station
Step 1. Go to the folder Arduino-ESP32-IDE and open the app arduino.exe (Fig. 1);

Figure 1. The archive with the Arduino IDE extracted to the folder

Step 2. Select New in the File menu (Fig. 1.1);

Figure 1.1. Create a new sketch

Step 3.1. Select Include Library->manage Libraries, in the Sketch menu and search for Simple DHT library and install (Fig. 1.2);

Step 3.1. Select Include Library->manage Libraries, in the Sketch menu and search and install M5Stack library and M5Stack-SD-Updater(Fig. 1.3);

Step 3.3. Select Include Library, M5Stack in the Sketch menu (Fig. 1.4);

Figure 1.4. Connection library M5Stack

Step 4. Prepare the temperature sensor and humidity DHT11 (Fig. 2),  Using a diagram in Figure 2.1 connecting DHT11 module, positive pin to 5V , negative pin to GND and data pin (middle) directly  to port 5 of M5Stack ( pull up resister  are provided in the module)

Figure 2. Temperature sensor and humidity DHT11

Figure 2.1 Connecting sensor to MCU (M5Stack)

Step 5. Write a simple sketch, which main task will be the output temperature in degrees Celsius on the display (if 27 and below, green if above, red color) (Fig. 3);

#include <M5stack.h>

#include <SimpleDHT.h>

#include <Wire.h>

#define DHTPIN 5

SimpleDHT11 dht;

int temperature, humidity;

int old_t = -1000;

void setup(){

M5.begin();

Wire.begin();

M5.Lcd.setTextSize(10);

}

void loop() {

float temp,humid;

int status = dht.read2(DHTPIN, &temp, &humid, NULL);

if (status == SimpleDHTErrSuccess) {

temperature = temp;

humidity = humid;

}

if (temperature != old_t){

if (temp > 27){

M5.Lcd.setTextColor(0xe8e4);

}

else{

M5.Lcd.setTextColor(0x2589);

}

M5.Lcd.fillScreen(0);

M5.Lcd.setCursor(0,0);

M5.Lcd.print(temperature);

M5.Lcd.print(“ C”);

}

old_t = temperature;

M5.update();

}

Figure 3. Our sketch

Step 5. Click the Upload button (Fig. 4) in order to flash the device;

Figure 4. Download the firmware to the device

Step 6. When the device firmware is completed, the screen will display the temperature (Fig. 5, 5.1) :)

Figure 5. Temperature 24 and 28 degrees Celsius

Task 2: Celcius Temp/ Hygrometer

Make a sketch to display both temperature and humidity including their unit on M5stack Lcd screen

Task 3: Fahrenheit Celcius Temp/ Hygrometer

Make a sketch to display both temperature and humidity including their unit on M5stack Lcd screen and  the  temperature unit shown can be changed by pushing button A.

Task 4: Hygrometer, Temp Clock

Make a sketch to display Hygrometer Celcius Temp Clock, where the temperature unit can be changed by button A, and the clock display mode (0-12 or 0-24) can be changed by pushing button B.

Section_____________date__________

Name____________________Student ID______________

Laboratory 4 Weather Station:

Task1: Basic weather station

Graded by _____________________Time________________

Task 2: Celcius Temp/ Hygrometer

Graded by _____________________Time________________

Task 3: Fahrenheit Celcius Temp/ Hygrometer

Graded by _____________________Time________________

Task 4 :Hygrometer, Temp Clock

Graded by _____________________Time________________

What do you learn from this lab?